| Block / Section | Exact recommended part (production) |
| --- | --- |
| Red LEDs (Tail/Brake strip) | ams OSRAM OSLON Signal – LJ CRBP.01-JZLX-27-3A4A-350-R18 |
| Amber LEDs (Turn/Hazard strip) | ams OSRAM OSLON SX – LA CN5M-FBGB-24-1-140-R18-Z |
| LED Driver (2 channels) | MPS MPQ7210-AEC1 (AEC-Q100) |
| MCU / Controller | Espressif ESP32-WROOM-32E (e.g., ESP32-WROOM-32E-N4) |
| 12V → 5V buck (Automotive) | TI LM53603AQPWPTQ1 |
| 5V → 3.3V LDO (Automotive) | TI TLV76733QWDRBRQ1 |
| Fuse holder (inline, sealed) | Littelfuse FHAC0001ZXJ |
| Fuse (example safe choice) | Littelfuse 0997005.WXN (Mini blade, 5A) |
| Reverse polarity protection | Nexperia BUK6Y24-40P (AEC-Q101) |
| Battery transient / surge clamp (TVS) | SMBJ58A (or automotive “-Q” variant like SMBJ58A-Q) |
| String balancing resistors (per parallel string) | Yageo PT2512FK-7W0R47L (0.47Ω, AEC-Q200) |
| Power inductor (per buck channel) | Coilcraft XAL7030-103MEB (10µH, AEC-Q200 family) |

Production BOM (recommended parts):

Prototype BOM (Best selections from in-house stock):

| Sr. | Block / Section | Best selection (from your stock) | Purpose / Use in prototype | Notes (important) |
| --- | --- | --- | --- | --- |
| 1 | MCU + Bluetooth | ESP32-WROOM-32 | Main controller; reads RUN/BRAKE/TURN buttons; generates PWM & blink | Best choice for fast prototyping (PWM + BLE) |
| 2 | LEDs (available stock) | WS2815 RGBW (12V type) | Visual demo of patterns + logic | If the project is single-color red/amber, this is only for demo. Real brightness/thermal won’t match final design |
| 3 | LED Driver IC (stock) | CAT4101 | Constant current driving for small LED test loads | Useful for controlled current tests, but not a full 12V buck driver for big strip power (thermal/architecture limits) |
| 4 | Power MOSFET | AO3400A | Low-side switching for Red/Amber channels (PWM from ESP32) | Best among listed for switching loads in prototype |
| 5 | Buck Converter | TPS54202 | 12V → 5V for ESP32 and logic | Cleaner and more reliable vs LM2596 class for prototyping |
| 6 | Reverse Polarity Protection | PMOS reverse protection (preferred) | Protects if supply polarity is reversed | Better than SS34 (less voltage drop); SS34 can be fallback |
| 7 | TVS (Transient suppression) | SM8S36A | Clamps surges/spikes on 12V input | Strongest clamp option among your TVS list (higher surge capability) |
| 8 | Input Signal Conditioning (optional) | TLV2372 | Buffer/condition brake/turn/running inputs | For office buttons, not required; helpful later for noisy vehicle signals |
| 9 | Current Sense Resistor | 0.1Ω 1% | Measuring current during prototype + basic protection | Easier to measure voltage drop than 0.05Ω; still low loss |
| 10 | Bluetooth Antenna | PCB trace antenna (ESP32 module) | Wireless comms | Simplest + reliable for prototype |
| 11 | Crystal Oscillator | Internal (ESP32 module) | Clock | External crystal usually not needed if using ESP32 module |
| 12 | Power Connector | Screw Terminal | Easy bench wiring for 12V input + LED outputs | Best for lab prototype (quick connect/disconnect) |
| 13 | Programming Interface | CP2102 | Flashing/debugging | Most stable/common option |
| 14 | PCB Substrate | FR4 TG170 | PCB reliability/temperature margin | Better temp margin than TG150 for automotive-like environment |